Routine name:	initdump

Description:	Initializes the simulation dumping/undumping system.  Normally

generated by default in a dumpfile, but it can be overridden

from the script to set various flags.

Usage:		initdump -version # -ignoreorphans # -allowmsgdup

-version #: The version number of the dumpfile. Allows it to

read old dumpfiles. Each dumpfile keeps its version number in

its header. This is relevant only for reading in dumpfiles, as

simdump always dumps using the latest version.

-ignoreorphans #: # can be 0 or 1. Orphans are elements

without a parent. If ignoreorphans is 1, then simundump will

simply skip over orphan elements. Otherwise it will complain.

-allowmsgdup: When a dumped simulation is being loaded onto

an existing one, then some messages are likely to be duplicated.

Normally such duplicate messages are detected and are NOT

created. This flag disables the mechanism that protects

dumps from duplicating msgs when the elements specified in

the dumps overlap.

Example:	Here is a little dumpfile using initdump that recreates a

simple 2-compartment model, and ignores the orphan element

/x/y/z.

============================================================

//genesis

initdump -version 3 -ignoreorphans 1

simobjdump neutral

simobjdump compartment activation Vm previous_state \

Im Em Rm Cm Ra inject dia len initVm

simundump neutral /x/y/z 0

simundump neutral /a 0

simundump compartment /a/compt 0 0 0.6632976405 0.6632942696 \

-0.3333315551 0 1 1 1 1 0 0 0

simundump neutral /b 0

simundump compartment /b/compt 0 0 0.3299660931 0.3299627243 \

0.3333349228 0 1 1 1 0 0 0 0

addmsg /b/compt /a/compt RAXIAL Ra Vm

addmsg /a/compt /b/compt AXIAL Vm

enddump

// End of dump

============================================================

Notes:

See also:       enddump, simdump, simobjdump, simundump